# Research Proposal: Optimizing Communication in Drone Ad-hoc Networks using Reinforcement Learning

# Primary Researcher

Prof. Reuven Cohen (Technion)

# Co-researcher

Barak Gahtan (PhD student, Technion)

# 1. Introduction

Unmanned Aerial Vehicles (UAVs), commonly known as drones, are increasingly deployed for real-time monitoring, surveillance, and disaster-response operations. In such applications, fleets of drones must communicate efficiently with a control center (ground station), often beyond direct communication range.

This results in a dynamic, multi-hop aerial network known as a Flying Ad-hoc Network (FANET). Ensuring reliable and efficient communication in such environments is challenging due to mobility, topology changes, and limited wireless resources.

This proposal focuses on optimizing communication in such networks by jointly designing routing strategies and adaptive topology control using reinforcement learning (RL).

# 2. System Model

We consider a heterogeneous drone network consisting of two types of drones:

- M-Drones (Mission Drones): These drones execute mission tasks (e.g., sensing, imaging) and also participate in relaying communication.

- C- Drones (Communication Drones): These drones are dedicated to communication support. They can dynamically reposition themselves to improve connectivity, reduce congestion, and enhance overall network performance.

All drones cooperate to ensure reliable communication between M-drones and the ground station.

# 3. Problem Definition

The goal of this research is to optimize end-to-end communication performance between M-drones and the ground station in a highly dynamic network. This involves two coupled challenges:

- Efficient Routing: Determining how data should be routed from each M-drone to the ground station over multiple hops.

- Topology Optimization: Dynamically controlling the positions of C- drones to ensure fault-tolerance, improve connectivity, reduce bottlenecks, and enhance link quality.

Unlike traditional approaches, we explicitly decouple mission execution (M-drones) from communication optimization (C- drones), enabling a more flexible and controllable system design.

# 4. Proposed Approach

We propose a distributed multi-agent framework consisting of two types of cooperating agents:

- Routing Agent:

  - Deployed on each M-drone and C-drone.

  - Responsible for selecting routes from the local drone to the ground station.

  - Operates based on locally observable network metrics such as link quality, congestion, delay, and hop count.

  - Adapts routing decisions to changing network conditions in a decentralized manner.

- Topology Agent:

  - Deployed on each C- drone.

  - Controls the movement of its host drone.

  - Optimizes its position relative to neighboring drones to improve network connectivity and performance.

  - Aims to reduce path length, avoid network partitions, and balance traffic load through local decisions.

The two agent types exhibit bidirectional influence:

- The topology agents collectively shape the network graph through distributed repositioning of C-drones.

- The routing agents exploit the resulting topology to select efficient paths.

Importantly, the system is fully distributed: there is no centralized controller. Each agent operates based on local observations and limited network information, enabling scalability, robustness, and adaptability to highly dynamic environments.

This separation allows scalable optimization while maintaining coordination between communication structure and routing decisions.

# 5. Expected Contributions

- A novel framework for communication optimization in UAV ad-hoc networks using heterogeneous drone roles.

- A multi-agent separating routing and topology control.

- Algorithms for dynamic placement of communication drones (C-drones).

- A simulation environment for evaluating large-scale UAV communication systems.

- Demonstrated improvements in throughput, latency, and network resilience compared to baseline approaches.

# References

1. 'Spectral RL–based Dynamic Routing for UAVNETs', Ad Hoc Networks, 2025.
2. 'IQMR: Improved Q-learning Multi-hop Routing for UAV Networks', arXiv, 2024.
3. 'UAV Immersive Video Streaming: A Comprehensive Survey', arXiv, 2023.